Samuel Lichtenheld

10/29/15

Assignment 3

Introduction

This assignment covered the single cycle mips architecture. The basic datapath was implemented, preparing us to implement a pipeline later.

Instruction Functionality

| Name | Operation | Type |
| --- | --- | --- |
| Add | R[rd] = R[rs] + R[rt] | R |
| add imm. | R[rt] = R[rs] + SignExtImm | I |
| add imm. uns. | R[rt] = R[rs] + ZeroExtImm | I |
| add uns. | R[rd] = R[rs] + R[rt] | R |
| and | R[rd] = R[rs] & R[rt] | R |
| and imm. | R[rt] = R[rs] & ZeroExtImm | I |
| branch eq | if (R[rs]==R[rt]) PC=PC+4+BranchAddr | I |
| branch not eq | if (R[rs]!=R[rt]) PC=PC+4+BranchAddr | I |
| jump | PC=JumpAddr | J |
| jump and link | R[31]=PC+8;PC=JumpAddr | J |
| jump reg | PC=R[rs] | R |
| load byte uns. | R[rt] = {24’b0,M[R[rs]+SignExtImm](7:0)} | I |
| load halfword uns. | R[rt] = {16’b0,M[R[rs]+SignExtImm](15:0)} | I |
| load upper imm. | R[rt] = {imm, 16’b0} | I |
| load word | R[rt] = M[R[rs]+SignExtImm] | I |
| Nor | R[rd] = ~(R[rs] ¦ R[rt]) | R |
| Or | R[rd] = R[rs] ¦ R[rt] | R |
| Or imm. | R[rt] = R[rs] ¦ ZeroExtImm | I |
| set less than | R[rd] = (R[rs]<R[rt]) ? 1:0 | R |
| set less than imm. | R[rt] = (R[rs]<SignExtImm) ? 1:0 | I |
| set less than imm. uns. | R[rt] = (R[rs]<SignExtImm) ? 1:0 | I |
| set less than uns. | R[rd] = (R[rs]<R[rt]) ? 1:0 | R |
| shift left logical | R[rd] = R[rt] << shamt | R |
| shift right logical | R[rd] = R[rt] >>> shamt | R |
| store byte | M[R[rs]+SignExtImm}(7:0) = R[rt](7:0) | I |
| store halfword | M[R[rs]+SignExtImm}(15:0) = R[rt](15:0) | I |
| store word | M[R[rs]+SignExtImm} = R[rt] | I |
| subtract | R[rd] = R[rs] -R[rt] | R |
| subtract uns. | R[rd] = R[rs]= R[rt] | R |

Implentation Order

# Set1

All R-type instructions except jr

add

addi

addu

and

or

nor

slt

sltu

sll

srl

sub

subu

# Set2

beq

bne

jal

jr

j

Set3

lbu

lhu

lw

sb

sh

sw

Figure 1: Everything but Load and Stores without PC

Figure 2: Everything PC related (jumps and branches)

Figure 3: Byte Addressable Mem

Figure 4: Byte Addressable Mem Control Signals (Little Endian)

# ROP_TEST

addi $1, $0, 0x0001

addi $2, $0, 0xffff

add $3, $2, $1

addu $3, $2, $1

sub $3, $2, $1

sub $3, $1, $2

lui $1, 0xffff

and $3, $1, $2

or $3, $1, $2

sltu $3, $0, $1

sll $3, $3, 16

srl $3, $3, 16

Branch & Jump testing through Fibonacci Sequence

0:	addiu $1, $0, 0x0005

1:	add $4, $0, $0

2:	add $2, $r0, $0

3:	addi $5, $0, 0x0001

for:

4:	slt $10, $2, $1

5:	beq $10, $0, exit(9)

6:	slti $10, $2, 0x0002

7:	beq $10, $0, else(2)

8:	add $3, $2, $0

9:	j forloopend

else:

a:	add $3, $4, $5

b:	add $4, $5, $0

c:	add $5, $3, $0

forloopend:

d:	addi $2, $2, 0x0001

e:	j for

exit:

f:	j exit

Figure 5: jumps exhibited

Figure 6: BEQ if equality isn't satisfied

Figure 7: BEQ if equality is satisfied

Byte Addressable Ram Testing

0:	lui $1, 0x0100

1:	lui $2, 0xFFFF

2:	lui $3, 0x5555

3:	lui $4, 0x8888

4:	addi $2, 0xFFFF

5:	addi $3, 0x5500

6:	addi $4, 0x0088

7:	addi $5, 0xDDDD

8:	addi $6, 0x0001

9:	addi $7, 0x0012

a:	addi $8, 0x0023

b:	sw $1, 0($0)

c:	sw $2, 4($0)

d:	sw $3, 8($0)

e:	sw $4, 12($0)

f:	lw $20, 12($0)

10:	lw $21, 8($0)

11:	lw $22, 4($0)

12:	lw $23, 0($0)

13:	sw $1, 0($6)

14:	sw $2, 4($6)

15:	sw $3, 8($6)

16:	sw $4, 12($6)

17:	lw $20, 12($6)

18:	lw $21, 8($6)

19:	lw $22, 4($6)

1a:	lw $23, 0($6)

….Continued with various test cases on all edges

Waveforms on next page